The 4 Xli files released for download are:

All Xli's use the following files:

192-8000.bt

192-8001.bin

192-8002.fs

The 184-8000.bin file, this is the FPGA data.

On Xli's shipped after March 2005 we now allow the FPGA code to be software downloadable in the file 184-8000.bin. This file needs to be updated.

On Xli's purchased before March 2005, you cannot upgrade the 184-8000.bin.  FPGA less than FPGA # 184-8000V50 cannot be updated.

To verify the FPGA version,

Hit the Function 18 and proceed to the FPGA screen. If the FPGS is FPGA # 184-8000V50

or higher the FPGA file needs to be updated. If it is not than the FPGA cannot be updated.

See XLi manual for complete upgrade instructions.

Below is a copy of a telnet session with the current firmware doing a successful firmware download.

WELCOME TO SYMMETRICOM NETWORK INTERFACE!
USER NAME: operator
PASSWORD: *****
NETWORK INTERFACE 192-8001      (c) 1998 - 2004 SYMMETRICOM
ALL RIGHTS RESERVED
LOGIN SUCCESSFUL!

>f100 bh 192.168.48.48 192-8000.bt
BURN HOST IS READY!!!
>
>f100 bub
OK

BURNING FILE 192-8000.bt WITH SIZE 467776 TO PARTITION:0 SECTOR:0

FILE: 467776 BYTES, PARTITION: 655360 BYTES (71% used)
SEC: 0 RE: 0
SEC: 1 RE: 0
SEC: 2 RE: 0
SEC: 3 RE: 0
SEC: 4 RE: 0
SEC: 5 RE: 0
SEC: 6 RE: 0
SEC: 7 RE: 0
FLASH SUCCESSFULLY PROGRAMMED CRC32 = 0x36E10DC5

>f100 K I L L
OK
RESETTING THE UNIT
PLEASE WAIT...

>

WELCOME TO SYMMETRICOM NETWORK INTERFACE!
USER NAME: operator
PASSWORD: *****
NETWORK INTERFACE 192-8001      (c) 1998 - 2004 SYMMETRICOM
ALL RIGHTS RESERVED
LOGIN SUCCESSFUL!

>f100 bh 192.168.48.48 192-8001.bin
BURN HOST IS READY!!!
>f100 bu
OK

BURNING FILE 192-8001.bin WITH SIZE 982672 TO PARTITION:1 SECTOR:16

FILE: 982672 BYTES, PARTITION: 5112320 BYTES (19% used)
SEC: 16 RE: 0
SEC: 17 RE: 0
SEC: 18 RE: 0
SEC: 19 RE: 0
SEC: 20 RE: 0
SEC: 21 RE: 0
SEC: 22 RE: 0
SEC: 23 RE: 0
SEC: 24 RE: 0
SEC: 25 RE: 0
SEC: 26 RE: 0
SEC: 27 RE: 0
SEC: 28 RE: 0
SEC: 29 RE: 0
SEC: 30 RE: 0
FLASH SUCCESSFULLY PROGRAMMED CRC32 = 0x1055B42B

>f100 bh 192.168.48.48 192-8002.fs
BURN HOST IS READY!!!
>f100 bf
OK
BURNING FILE 192-8002.fs WITH SIZE 524288 BYTES (  25% used)
SEC: 94
SEC: 95
SEC: 96
SEC: 97
SEC: 98
SEC: 99
SEC: 100
SEC: 101
SEC: 102
FILE SYSTEM FLASH BURN COMPLETED

>f100 K I L L

# To update the FPGA

To burn the 184-8000.bin enter: (if required)

F100  bh  <_ip_address> 184-8000.bin

F100 bufp

FLASH SUCCESSFULLY PROGRAMMED

Then enter “K (space) I (space) L (space) L” as shown here:

F100 K I L L

The “K I L L” command reboots your unit.