The 4 Xli files released for download are:

All Xli's use the following files:

192-8000.bt

192-8001.bin

192-8002.fs

The 184-8000V64.bin file, this is the FPGA data.

On Xli's shipped after March 2005 we now allow the FPGA code to be software downloadable in the file 184-8000V64.bin. This file needs to be updated.

On Xli's purchased before March 2005, you cannot upgrade the 184-8000.bin.  FPGA less than FPGA # 184-8000V50 cannot be updated.

To verify the FPGA version,

Hit the Function 18 and proceed to the FPGA screen. If the FPGS is FPGA # 184-8000V50

or higher the FPGA file needs to be updated. If it is not than the FPGA cannot be updated.

See XLi manual for complete upgrade instructions.

Below is a copy of a telnet session with the current firmware doing a successful firmware download.

WELCOME TO SYMMETRICOM NETWORK INTERFACE!
USER NAME: operator
PASSWORD: *****
NETWORK INTERFACE 192-8001      (c) 1998 - 2008 SYMMETRICOM
ALL RIGHTS RESERVED
LOGIN SUCCESSFUL!

>f100 bh 192.168.49.7 192-8000.bt

BURN HOST IS READY!!!

>f100 bub

OK

BURNING FILE 192-8000.bt WITH SIZE 485788 TO PARTITION:0 SECTOR:0

FILE: 485788 BYTES, PARTITION: 655360 BYTES (74% used)

SEC: 0 RE: 0

SEC: 1 RE: 0

SEC: 2 RE: 0

SEC: 3 RE: 0

SEC: 4 RE: 0

SEC: 5 RE: 0

SEC: 6 RE: 0

SEC: 7 RE: 0

FLASH SUCCESSFULLY PROGRAMMED CRC32 = 0xA211E1B0

>f100 bh 192.168.49.7 192-8001.bin

BURN HOST IS READY!!!

>f100 bu

OK

BURNING FILE 192-8001.bin WITH SIZE 1027432 TO PARTITION:1 SECTOR:16

FILE: 1027432 BYTES, PARTITION: 5112320 BYTES (20% used)

SEC: 16 RE: 0

SEC: 17 RE: 0

SEC: 18 RE: 0

SEC: 19 RE: 0

SEC: 20 RE: 0

SEC: 21 RE: 0

SEC: 22 RE: 0

SEC: 23 RE: 0

SEC: 24 RE: 0

SEC: 25 RE: 0

SEC: 26 RE: 0

SEC: 27 RE: 0

SEC: 28 RE: 0

SEC: 29 RE: 0

SEC: 30 RE: 0

SEC: 31 RE: 0

FLASH SUCCESSFULLY PROGRAMMED CRC32 = 0x810D0CD3

>f100 K I L L

OK

RESETTING THE UNIT

PLEASE WAIT...

WELCOME TO SYMMETRICOM NETWORK INTERFACE!
USER NAME: operator
PASSWORD: *****
NETWORK INTERFACE 192-8001      (c) 1998 - 2008 SYMMETRICOM
ALL RIGHTS RESERVED
LOGIN SUCCESSFUL!

>f100 bh 192.168.49.7 192-8002.fs

BURN HOST IS READY!!!

>f100 bf

OK

BURNING FILE 192-8002.fs WITH SIZE 2096640 BYTES

SEC: 94

SEC: 95

SEC: 96

SEC: 97

SEC: 98

SEC: 99

SEC: 100

SEC: 101

SEC: 102

SEC: 103

SEC: 104

SEC: 105

SEC: 106

SEC: 107

SEC: 108

SEC: 109

SEC: 110

SEC: 111

SEC: 112

SEC: 113

SEC: 114

SEC: 115

SEC: 116

SEC: 117

SEC: 118

SEC: 119

SEC: 120

SEC: 121

SEC: 122

SEC: 123

SEC: 124

SEC: 125

FILE SYSTEM FLASH BURN COMPLETED

>f100 K I L L

OK

RESETTING THE UNIT

PLEASE WAIT...

# To update the FPGA

To burn the 184-8000V64.bin enter: (if required)

F100  bh  <_ip_address> 184-8000V64.bin

F100 bufp

FLASH SUCCESSFULLY PROGRAMMED

Then enter “K (space) I (space) L (space) L” as shown here:

F100 K I L L

The “K I L L” command reboots your unit.