Upgrading System Firmware

Caution: Consideration must be given to the firmware and the hardware version numbers of the XLi and it’s optional com-ponents. Consult with Symmetricom’s Customer Service department before performing upgrades.

This section explains how to completely upgrade the system firmware. This is done using the F100 BUFP,  F100 BH, F100 BUB, F100 BU, and F100 BF commands.

Overview of Procedure

- Set up a network connection between your XLi and the FTP server.

- Set up an FTP Server with the firmware upgrade files.

- Open a command line session to the XLi.

- Install the firmware version 8 files.

- Reboot the XLi.

Note:If your system’s NTP and SNMP configuration files (ntp.conf, ntp.keys, snmp.conf) have been customized, make backup copies and later re-install those configuration files. See “A: Using F100 Configuration” in the Xli manaul

Set up the FTP Server

To save time and trouble, download a preconfigured FTP server from http://www.ntp-systems.com/zip/ warftpd1.zip and extract it to the C:\ drive on your workstation. Otherwise, customize your existing FTP server setup as described in this section.

When performing these operations, the user issues command line instructions to the XLi. The XLi responds to those commands by connecting to the FTP server and burning the software to system memory. The XLi gives the FTP server ‘Anonymous’ as its user name, and uses a null password (e.g., the equivalent of pressing the Enter key on your keyboard instead of entering text). The FTP server must be configured as follows:

- Anonymous log-ins are enabled

- The password for Anonymous is disabled, or allows a null password

- Anonymous has read/write privileges to Anonymous’s home directory.

Obtain the current system firmware files (E.g., 184-8000.bin 192-8001.bin, 192-8002.fs) from Symmetricom’s customer support website.

Place the system firmware upgrade files in home directory of the ‘anonymous’ user. If you’re using the preconfigured FTP server, C:\TrueTime is the default or home directory.

Open a Command Line Session on the XLi

Note: The XLi and FTP server need to be connected by a TCP/IP network. Ideally they should be on an isolated subnet. Connecting them over a network with multiple ‘hops’ or one with heavy network traffic raises the possibility that the system software files could be corrupted, yielding the XLi inoperable.

Telnet to the XLi over the network. For example, at your workstations command line, type “telnet 192.168.46.10”.

When prompted, log in to the unit using the operator’s username and password; the user name and password are usually “operator” and “janus”.

Upgrade the Firmware

Command Format

When issuing the firmware upgrade commands, use the following format:

F100 <command> <ftp_server_ip_address> <relative_path>/<file.ext>

If the FTP server is on your workstation, <ftp_server_ip_address> is the IP address of your workstation.

<relative_path> is a subdirectory inside the anonymous user’s home directory on the FTP server. If there is no subdirectory (i.e., if the upgrade files are sitting in anonymous user’s home directory), drop <relative_path> from the command line.

For example, if c:/ftpworkfiles is the anonymous user’s home directory, and the upgrade files are in c:/ ftpworkfiles/xli/, you would enter the command as follows:

To burn the 184-8000.bin enter:

F100  bh  <_ip_address> 184-8000.bin

F100 bufp

Then enter “K (space) I (space) L (space) L” as shown here:

F100 K I L L

The “K I L L” command reboots your unit.

To burn the  192-8001.bin file enter:

F100 bh <IP_address> <relative_path>/<file.bin>

For example:

F100 bh 192.168.49.120 /192-8001.bin

The XLi responds:

BURN HOST IS READY<CR><LF>

Then enter:

F100 bu

The XLi responds:

OKBURNING FILE 192-8001.bin WITH SIZE 803016 TO PARTITION:1 SECTOR:10SEC: 10 RE: 0SEC: 11 RE: 0SEC: 12 RE: 0SEC: 13 RE: 0SEC: 14 RE: 0SEC: 15 RE: 0SEC: 16 RE: 0SEC: 17 RE: 0SEC: 18 RE: 0SEC: 19 RE: 0SEC: 20 RE: 0SEC: 21 RE: 0SEC: 22 RE: 0FLASH SUCCESSFULLY PROGRAMMED CRC32 = 0x2D9A260A

To burn the 192-8002.fs file enter:

F100 bh <IP_address> <relative_path>/<file.fs>

For example:

F100 bh 192.168.49.120 /192-8002.fs

The XLi responds:

BURN HOST IS READY<CR><LF>

Then enter:

F100 bf

The XLi responds:

OKBURNING FILE 192-8002.fs WITH SIZE 524288SEC: 94SEC: 95SEC: 96SEC: 97SEC: 98SEC: 99SEC: 100SEC: 101SEC: 102

Then enter “K (space) I (space) L (space) L” as shown here:

F100 K I L L

The “K I L L” command reboots your unit. You have completed the firmware upgrade procedure

Troubleshooting

Most problems upgrading the firmware are due to problems with the configuration of the FTP server, such as:

- setting the server to accept a null password

- configuring the anonymous home directory

- setting the correct access rights

- entering the correct relative file pathThe following error messages may provide some indication of the underlying problem:

Message: >Can't set the burn host - wrong IP address

Cause: The IP address entered for the FTP server is incorrect. Check that you've entered the IP address of the FTP server (not the XLi) and re-enter if necessary.

Message: >Can't open file: 192-####.##

Cause: There's a problem with the FTP server that is preventing access to the file. Verify the following:

- The FTP server is correctly configured.

- The anonymous user account is enabled.

- The anonymous user account password is "guest".

- The anonymous user account has read access to the ftpworkfiles directory.

- The ftpworkfiles directory located in the anonymous user's home directory.

- The FTP server is running.

- There aren’t any other ‘anonymous’ users logged into the FTP server.

Try connecting to the FTP server as ‘anonymous’ using an FTP client. You should automatically see the product name directory (e.g., “XLi”) you created inside the anonymous users home directory (e.g., “c:\ftpworkfiles\”). Open the product name directory. You should see the firmware upgrade files you put there. If either the product name directory or the firmware upgrade files aren’t visible, there’s a problem with the FTP configuration.

Message: >Wrong File type

You may be using the wrong firmware files for the product being upgraded. This may be due to the incorrect files being place in the upgrade directory. It may also be that the wrong directory was entered (one for another product) in the path information on the F100 command line. ‘Wrong file type’ is also associated with ‘Can’t open file’ errors - see the preceding message.

Message: Unit hangs on “Burning Boot” message.

Check that your IP Address, Subnet Mask, and Default Gateway of the XLi are correctly configured.

FAQ

How does one check the unit’s firmware version number?

Log on to the XLi and enter the following command:

> F100 VER

An example XLi response is:

F100 VERBOOTLOADER 192-8000SOFTWARE 192-8001FILE SYSTEM 192-8002v1.80NVRAM VER 5PROJ REV #1.80

The “PROJ REV” number is the firmware version number. The “v” number in “FILE SYSTEM” is the file system version number, which may not be the same as the firmware version number.

How does one check the IP address, subnet mask, and default gateway of theXLi?

Log on to the XLi and enter the following command:

> F100 ICF100 IP:192.168.47.156 SM:255.255.255.0 G:192.168.47.1

Is the null modem cable necessary? What if I’m upgrading a XLi remotely?

The null-modem cable is optional. If you decide to Telnet to the XLi over TCP/IP network, you don’t need the null modem cable.

I’m using a null modem cable to connect to the XLi from my laptop and the XLi keeps rebooting?

An ungrounded voltage level on one of the pins in the null modem cable causes the unit to reset. Use one of the following work-arounds:

- Connect the laptop to a grounded power supply, if it has one, or ground the laptop’s chassis.

- Do away with the null modem cable. Telnet to the unit over the network.

Use a regular PC instead of the laptop. The PC is connected to a grounded power supply and doesn’t cause this problem.