E1938A Pinout Pictorial

Rick Karlquist, Dec. 10, 2004

Introduction

The E1938A oven crystal oscillator controller board has various configurations for its D-subminiature connector.  All configurations use

the same PC board with the same physical pin locations, however the

connector may be installed on either side of the board, and there are

various numbering schemes.  This pictorial document shows where various signals on physically located in a photo, to avoid ambiguity with connector pin numbering schemes.  Pin numbers in this document are the same as on the schematic.

Component side of board

See figure 1:

1. ACOM/DCOM

2. Vref Common

3. EFC shield

4. TX, serial port

5. Data from PIC

7. PIC data ready

6. Data to PIC

8. Vref, +2.5V

9. 10 MHz, common

10. 10 MHz, signal common

11. ACOM/DCOM

12. ACOM/DCOM

13. EFC+

14. +5V

15. +5V

16. RX, serial port

17. +12V

18. PIC clock

19. Not used

20. Not used

Figure 1.  Component side of E1938A controller board.

Circuit side of board

Note that pin numbers are embossed on this connector.  See figure 2:

5. Data from PIC

7. PIC data ready

1. ACOM/DCOM

2. Vref Common

3. EFC shield

4. TX, serial port

6. Data to PIC

8. Vref, +2.5V

9. 10 MHz, common

10. 10 MHz, signal common

11. ACOM/DCOM

12. ACOM/DCOM

13. EFC+

14. +5V

15. +5V

16. RX, serial port

17. +12V

18. PIC clock

19. Not used

20. Not used

Figure 2.  D-sub with coax insert on circuit side of PCB.